# Ontwikkeling van een sequenties bibliotheek

# Omschrijving

Omdat de standaard sequenties bibliotheek van ABB ondermaats is, is de ontwikkeling van een sequentie bibliotheek een must.

De bibliotheek van Nedalco wordt als voorbeeld genomen. Seq_Typical en Sequence Typical

# SibelcoSequentieLib

Voor elke sequentie moet een nieuw Control Module Type gemaakt worden.

# Seq_typical

Eerst bouwen van de code en nadien maken van de faceplates. In de SibelcoSequentiesLib is een Seq_Typical aangemaakt.

#### Parameters van Seq_Typical

| Name | Data Type | Description |
| --- | --- | --- |
| Name | String[30] | Naam van de sequentie |
| Description | String[60] | Omschrijving van de sequentie |
| ObjPar | sibSeqPar |  |
| AEClass | Dint | Alarm en Event klasse |
| A_Severity | Dint | Alarm severity |
| E_Severity | Dint | Event severity |

#### sibSeqPar data Type

| Name | Data Type | Description |
| --- | --- | --- |
| Start | Bool | IN: Start sequence |
| StartHSI | Bool | IN: Start from HSI |
| Block | Bool | IN: Inhibit start |
| EMSD | Bool | IN: reset sequence from emsd |
| AbortSequence | Bool | IN: Abort command from other sequences |
| Active | Bool | OUT: Sequence is active step > 5 |
| LastStepActive | Dint | OUT: Last active step before reset of sequence |
| SeqID | Dint | OUT: Unique ID of this sequence |
| ReleaseCode | Dint | IN: release from main sequence, -1 = undefined |
| ReadyCode | Dint | OUT: 1=ready, 0=in progress, 1=not ready |

De Seq_Typical bevat meerdere control modules / functions:

- sibElapsedTime1 (sibElapsedTime)

- sibElapsedTime2 (sibElapsedTime)

- sibElapsedTime3 (sibElapsedTime)

- sibElapsedTime4 (sibElapsedTime)

- Steps

  - Step000

  - Step010

  - StepXXX

- Settings

  - Timer1

  - Timer2

  - Timer3

  - Timer4

- SeqTime

- StepTime

- AlarmInd1

- AlarmInd2

- sibStringAddDint_1

- EvSeq

- RstB16

# sibElapsedTime (control module type)

#### Parameters van sibElapsedTime

| Name | Data Type | Description |
| --- | --- | --- |
| Name | String[30] | Naam |
| Description | String[40] | Omschrijving |
| TimeToElapse | Real |  |
| IaPar | sibTmerPar | Interaction par |
| Mode | Dint | 1=sec, 2=min |

#### sibTimerPar (data Type)

| Name | Data Type | Description |
| --- | --- | --- |
| Reset | Bool | IN: Resets the timer. Start command is set to false and time and offset or set to zero |
| Start | Bool | IN: (Re)Starts the timer |
| TimeHasElapsed | Bool | OUT: The time >= TimerSetting |
| RemainingTime_time | Time | OUT: Remaining Time |
| RemainingTime_ms | Dint | OUT: Remaining Time |
| ElapsedTime | Time | OUT: Elapsed Time |
| OffsetTime | Real | OUT: The time is added to the elapsed time. This offset is removed when a reset is perfomed. |